GUIDELINES FOR PREPARATION
 CERTIFICATE OF LOCATION OF GOVERNMENT CORNERS
Pursuant to section 3-4.01 of the Minnesota Department of Transportation hand book
 SURVEYING AND MAPPING MANUAL, dated April 20, 2007: (also see applicable statutes)

The Certificate of Location should be a complete document relating to the location, monumentation, perpetuation and history of a corner.  If done properly, the need for future surveyors to research the past record evidence will be greatly reduced or eliminated.  The following outline provides a guideline for preparation of a Corner Certificate.

- Corner Index System

  - Identify the lowest Township number common to the corner.

  - Identify the lowest Range number common to the corner.

  - Identify the lowest Section Number common to the corner.

  - Identify the corner location in the identified section. i.e., North Quarter, Northeast, East Quarter, etc.

  - Identify closing corners in the proper section.

  - If a corner is on a county line it should be identified and filed in each county.

  - List Section, Township, Range, Principal Meridian and County.

- Monument

  - Check the appropriate statement on the Certificate of Location for monument at corner location.

  - Indicate date of recovery and/or placement of monument.

  - Describe the monument in detail and give relationship to ground elevation.

  - If a monument is removed, during construction or for other purposes, explain fully understatement of evidence (on back of page).

- Sketch of Reference Ties

  - Make a minimum of two ties, more if practicable.

  - Identify the reference points in detail, e.g., nail and disc on S.W. side of (diameter) burr oak, distance above ground.

  - Reference tie distances are horizontal unless clearly stated on sketch.  Show difference in elevation and direction of slope for slope measurements.

  - Direction of reference point from corner should be given by:

    - Point of compass. i.e., N, NNE, NE, ENE, E, etc., or

    - By a bearing (preferred). i.e., N50°E.  For bearings indicate on sketch their basis of orientation and accuracy.

  - Show the improvements in the immediate vicinity such as roads, including name or number, buildings, fences, tree lines, retaining walls, field lines, and their relation to corner.

  - Indicate if corner is on a Correction Line.

- Statement of Evidence

  - Include all record evidence that relates to the history of this corner chronologically beginning with Public Land Survey.

  - Include all parol evidence and testimony concerning corner location.  List name, age, residence, address, job title, and how information was obtained.

  - Include all information on field search and excavation.  Indicate extent and depth of excavation; soil profile characteristics; present and original ground elevations could be added.

  - Include information on monument found in place.  Identify type and size of monument; who, when and how it was placed in addition to a photo.  Indicate in statement of evidence if no information is found on the origin or history of monument.

  - Method used to set lost or obliterated corners must be stated in a short summary with supporting evidence.

  - When a coordinate value is known, the coordinates may be listed.  If the coordinates are enumerated, the reference coordinate system must be indicated.

  - Analyze all of the information on the document.  Indicate the reasons for the decisions made in the form of a summary or conclusion statement.